ECE 415								Dr. H. El Naga

Verilog: Modeling, Simulation and Synthesis

Homework  #1

E/D

0

1

2

3

4

5

6

7

f

f

f

f

f

f

f

f

3 x 8

I0

I1

I2

1. a.	Write a Verilog structural model for 3x8 binary decoder. Use the following module header:

module ThreexEight ( …….    );

output ….;

input…..;

o o o

endmodule

- Write a well documented  exhaustive testbench to test the 3x8 binary decoder of part a.

module ThreexEight(f, i, enable);

output [7:0] f;

input [2:0] i;

input enable;

- Generate the displayed output, in the right order, for the following Verilog code. Indicate which system task is responsible for what output.

module display_example;

reg [15:0] data1, data2;

reg index;

initial

begin

$writeo (“%d” , $time, , “%h  \t”, data1, , “%h  \t”, data1, , index, “\n”);

#31 data1=15 ; data2 = 10; index = 1;

$displayh ($time, , data1, , data2, , index);

end

initial

begin

#5  data2 = 31 ; index = 0;

$strobe ($time , ,  data2 , , index);

$display ($time , , data2 , , index);

data2 = 15;

end

endmodule

X

C1

Y

C2

out

Complete the following table for the above circuit:

X    Y   C1   C2     out

x    x     0      0

x    1     0      1

0    x     1      0

x    0     0      x

1    x     x      0

0    1     x      x

4. a.	Write a structural model of a 4-input multiplexer by instantiating the 2-input mux of Lab #1: MUX2_1(out, a, b, sel).  Use the following module header:

module MUX4_1(out, In, sel);

output out;

input [3:0] In ;

input [1:0] sel;

o o o

endmodule

- Write a well documented exhaustive testbench to test the 4-input multiplexer of part a. Use hierarchical referencing to monitor every intermediate signal along the path from In[0] to the output.

5. a.	Using nand and xor Verilog primitives only, construct a full-adder module assuming the following:

nand gates  have a time delay of 10 ns each, and

xor gates have a time delay of 15 ns each.

Use the following module header:

module Full_adder ( cout, Sum, X, Y, cin);

output  cout;

output  Sum;

input   X, Y ;

input  cin;

o o o

endmodule

b.	Write a structural model of a 4-bit adder by instantiating the full adder module of part a (use an array of instances if possible).  Use the following module header:

module Adder_4( cout, Sum, X, Y, cin);

output cout;

output [3:0] Sum;

input [3:0] X, Y ;

input cin;

o o o

endmodule

- Write a testbench to test the 4-bit adder of part b. Use hierarchical referencing to monitor every intermediate signal along the path from X[2] to the output Sum[3].

6.	Are each of the following legal Verilog statement? If not, identify the error and suggest a correction for it.

a.  'timescale 1ns/1ns

b.  `timescale 50 ns / 10 ns;

c.  `timescale 100 ms /1 ns

d.  `timescale 1ns

e.  `timescale 1ns/100ps

module One (...);

o o o

endmodule

`timescale 10ns/1ns

module Two (...);

o o o

endmodule

- ` timescale 100fs/1ns

g.  ` t i m e s c a l e   1 0 0 n s / 1 0 n s

h.  `timecsale 10ns/1ns

i.  `timescale        100        ns        /        10        ns

j. `timescale 100ms/10ms

k. `timescale 100s/10s

l. `timescale 10ns/1ns

o o o

nand #10.0625 (...);